Major Features of Version 2.5: Dec 1, 1994

Built in HELP is available by pressing <F2> at most all screens.

Two basic types of HELP are available:

-  General HELP displays several lines of instructions

explaining what options are available at the current screen.

-  Individual help gives a 1 line definition of each spec or

output as you move the cursor around the menu or screen.

Both types of HELP give page numbers in the Version 2.5 Manual

for more information.

Built in EXAMPLES of specific parts (heads, cams, short blocks,

etc) are available by pressing <F3> at the 5 ENGINE SPECS menus.

This is very handy if you do not have flow bench data for heads.

Several features have been added to the plots for easier control

and adjustments, including Shift, Zoom, Gridline and Cursor

options.  Also, printing plots on a printer is much easier.  It

is also possible to save one plot as a Baseline with the Ctrl-B

command.  This one plot remains the Baseline until you use the

Ctrl-B command again or quit the program.

An Analysis Report is available with Ctrl-A when Projected

Performance has been calculated.  It gives suggestions for

improving performance, tips on how to interpret the results, and

points out SAFETY concerns.

Once you have calculated results, you can load the power curve

into a Performance Trends vehicle program, like DRAG RACING

ANALYZER v2.2 or CIRCLE TRACK ANALYZER v1.1.  Use the Ctrl-L

command to display a menu where you can specify what program to

Load and where it is located on the hard drive.  The ENGINE

ANALYZER will start up the vehicle program, and allow you to load

the power curve at your convenience.  When you quit the vehicle

program, control is transferred back to the ENGINE ANALYZER.

Several options are available to modify printouts of the results,

including printing a blank worksheet and a list of definitions

for all outputs.

V2.5 is more accurate at predicting effect of changing cam events

(Version 2.1 'liked' too much cam advance and early Exhaust

Opening).

V2.5's exhaust tuning simulation is more accurate.

In the CAM/VALVE TRAIN menu, CALCULATED SPECS of DURATION @ .050,

LOBE CENTERLINE, GROSS VALVE LIFT and LOBE SEPARATION are given.

This makes it easier to verify the cam specs you have entered

match most any cam card.

An EST IDLE VAC, ''Hg is now given in the results.

A built in tutorial is available to explain major features.

One new CONFIGURATION spec that has been added called BEGINNER

LEVEL HELP (Y/N).  The program comes with it set to Y assuming

you are new to computers or the program.  With BEGINNER LEVEL

HELP turned on, the program assumes you want to see the tutorial,

gives a definition of each spec on the Dialogue line, gives

additional explanation for some actions, etc.  For experienced

users, this may be bothersome, so set BEGINNER LEVEL HELP to N.

Other new CONFIGURATION specs allow you to:

-  Specify other printer ports and printer type than just LPT1.

-  Specify where to find the DOS program GRAPHICS and what

options to use with GRAPHICS.  This is important for

printing plots on a printer.

-  You now can specify Medium resolution, Monochrome plots

which appear better when printed.

Humidity is now identified by DEW POINT, DEG F rather than the 2

specs of RELATIVE HUMIDITY and REL HUM TEMP.  It is hoped this

method will be less confusing.  Calculation Menus are available

to calculate DEW POINT from either relative humidity and

temperature or wet & dry bulb temperatures.

Nitrous oxide specs now define a 2 stage nitrous system.  The

first stage comes on at RPM TO START NITROUS and delivers half of

the NITROUS OXIDE HP.  The second stage comes on at RPM FOR FULL

NITROUS and delivers the second half of the NITROUS OXIDE HP.

For a single stage system, set RPM TO START NITROUS and RPM FOR

FULL NITROUS to the same RPM, where the single stage nitrous

system is activated.

CFM in the Projected Performance has been changed to ACTCFM.  The

actual CFM is a more repeatable number from test to test with

different weather conditions.

The ENGINE LIBRARY now has 33 engines.

You can now specify FUEL OCTANE.  The program has replaced the

required OCTANE in the results with SPKADV.  Based on the OCTANE

you specify and the engine operating conditions at each RPM, the

program estimates the required spark advance.  If the engine is

likely to detonate (spark knock) with this spark and FUEL OCTANE,

the SPKADV is reduced (retarded) and the SPKADV # is listed with

an "r".  Torque and HP are also reduced appropriately due to the

retarded spark.

!!!  WARNING  !!!  Your actual required spark advance can be

quite different than this.  DO NOT use this spark curve to

set your distributor.  This value should only be used to see

trends.

When running ALCOHOL, the program now assumes a richer mixture,

5:1 A/F.  Other slight adjustments have been made in how the

program simulates an alcohol engine.

The L and X size turbos are slightly larger than if V2.1 to cover

a wider range of turbo applications.

Cam recommendations are now given to place the torque peak or HP

peak at certain RPMs.  This required some extra lines in the

output.  To save some space VALVE AVG FLOW COEF and VALVE

EXH/INT% were eliminated.  Also, VLV AREA, SQ IN*DEG now includes

the effect of the PORT & RUNNER DIA and RUNNER FLOW COEF.

The BASE ENGINE menu now calculates and prints the CURRENT ENGINE

DISP, CID and CURRENT CLEARANCE VOL, ccs based on the settings of

BORE, STROKE, # OF CYLINDERS and COMPRESSION RATIO.

The COMPRESSION RATIO calculation menu for milling the head was

eliminated.  It seemed to add confusion to the program for users.

If you specify a SUPERCHARGER, the INTAKE TYPE is marked 'na'.

The program assumes an INTAKE TYPE = 2 for the calculations.

The routine for entering and editing comments is more user

friendly.

ASCII files of the results can now be printed in either rows or

columns.

More explanation is given at several places in the program to

help the user understand their options.

The starting values in most Calculation Menus are now "zero'd

out".  This will hopefully eliminate confusion for users who did

not understand that these starting values were simply left over

from the last time the menu was used.

-------------  Version 2.5 Changes After March 22, 1995  --------

A bug was discovered in early v2.5s in March 1995, where increases

in Elevation would increase power.  This was corrected in versions

produced after March 22, 1995.

It was necessary to make a change in the Ctrl+L command to load

an engine file (torque curve) into another program, particularily

the CIRCLE TRACK ANALYZER.  Originally, the PROGRAM COMMAND in the

LOAD/RUN VEHICLE PROGRAM menu for the CIRCLE TRACK ANALYZER was

CTA.  Now you must use CTA11.  The PROGRAM COMMAND for the DRAG

RACING ANALYZER is still DRA.  The 'on screen' help is updated to

reflect these changes.