With PAW Cam

RPM      1400   1800   2200   2600   3000   3400   3800   4200   4600   5000

BRK TQ    191    201    206    212    216    217    210    195    178    160

BRK HP   50.8   68.8   86.3    105    123    140    152    156    156    152

EX PRS     .1     .3     .4     .6     .8    1.1    1.3    1.5    1.6    1.7

IN VAC     .3     .6     .6     .6     .6     .6     .6     .7     .8     .8

VE %     73.2   76.1   78.4   81.7   84.1   85.6   84.8   81.4   77.2   72.8

ACTCFM   75.0    100    126    155    185    213    236    250    260    266

FL FLW   26.0   34.8   43.8   54.0   64.1   74.0   81.9   86.9   90.3   92.5

BSFC     .512   .506   .508   .514   .520   .527   .538   .556   .579   .607

BSAC    6.765  6.684  6.706  6.783  6.867  6.960  7.106  7.343  7.642  8.009

SPKADV    18r    24r     29     29     29     30     30     30     31     31

------  VALVE FLOW & CAM CALCULATIONS  -------------------  --INT--  --EXH--

OVERLAP AREA, SQ IN*DEG   1.7          TOTAL AVG FLOW COEF  .259     .287

TOTAL EXH/INT %           82.7         LOBE AREA, INCH*DEG  20.74    22.72

LOBE SEPARATION, DEG      112.0        VLV AREA, DEG*SQ-IN  140.5    116.2

LOBE CENTRLINS, DEG  112.0    112.0

------  GENERAL ENGINE CALCULATIONS  ---------------------------------------

DISPLACEMENT, cc          3852.1       DISPLACEMENT, cu in       235.03

DYNAMIC COMP. RATIO       6.24         COMPRESSION RATIO         7.90

THEO. CRANK COMPRSSN,PSI  142          CLEARANCE VOLUME, ccs     93.0

CARB MINIMUM WOT RPM      180          EST IDLE VAC, ''Hg        18.5

INTAKE CAM SPECS:                  +------ CALCULATED SPECS -----+

OPEN @.050, BTC (C)     -10      ¦                             ¦

CLOSE @.050, ABC (C)    34       ¦ INTAKE:                     ¦

MAX LOBE LIFT, IN       .281     ¦   DURATION @ .050     204.0 ¦

LASH @ VALVE, IN        0        ¦   LOBE CENTERLINE     112.0 ¦

ROCKER ARM RATIO        1.6      ¦   GROSS VALVE LIFT    0.450 ¦

EXHAUST CAM SPECS:                 ¦                             ¦

OPEN @.050, BBC (C)     39       ¦ EXHAUST:                    ¦

CLOSE @.050, ATC (C)    -5       ¦   DURATION @ .050     214.0 ¦

MAX LOBE LIFT, IN       .296     ¦   LOBE CENTERLINE     112.0 ¦

LASH @ VALVE, IN        0        ¦   GROSS VALVE LIFT    0.474 ¦

ROCKER ARM RATIO        1.6      ¦                             ¦

ADDITIONAL CAM ADVANCE    0        ¦ LOBE SEPARATION       112.0 ¦

¦                             ¦