PAW 4 deg. Adv.

RPM      1400   1800   2200   2600   3000   3400   3800   4200   4600   5000

BRK TQ    195    204    210    216    219    218    208    192    174    155

BRK HP   52.0   70.1   88.0    107    125    141    150    153    153    147

EX PRS     .1     .3     .4     .6     .9    1.1    1.3    1.5    1.6    1.6

IN VAC     .3     .6     .6     .6     .6     .6     .6     .7     .8     .8

VE %     75.0   77.5   79.8   82.9   85.2   85.9   84.0   80.3   76.0   71.0

ACTCFM   76.8    102    128    158    187    214    234    247    256    260

FL FLW   26.7   35.5   44.6   54.8   65.0   74.2   81.2   85.7   88.8   90.3

BSFC     .513   .506   .507   .513   .519   .527   .540   .559   .582   .612

BSAC    6.770  6.681  6.692  6.767  6.851  6.959  7.124  7.375  7.683  8.085

SPKADV    17r    23r    28r     29     29     30     30     30     31     31

------  VALVE FLOW & CAM CALCULATIONS  -------------------  --INT--  --EXH--

OVERLAP AREA, SQ IN*DEG   1.7          TOTAL AVG FLOW COEF  .259     .287

TOTAL EXH/INT %           82.7         LOBE AREA, INCH*DEG  20.74    22.72

LOBE SEPARATION, DEG      112.0        VLV AREA, DEG*SQ-IN  140.5    116.2

LOBE CENTRLINS, DEG  108.0    116.0

------  GENERAL ENGINE CALCULATIONS  ---------------------------------------

DISPLACEMENT, cc          3852.1       DISPLACEMENT, cu in       235.03

DYNAMIC COMP. RATIO       6.43         COMPRESSION RATIO         7.90

THEO. CRANK COMPRSSN,PSI  149          CLEARANCE VOLUME, ccs     93.0

CARB MINIMUM WOT RPM      180          EST IDLE VAC, ''Hg        18.5

INTAKE CAM SPECS:                  +------ CALCULATED SPECS -----+

OPEN @.050, BTC (C)     -10      ¦                             ¦

CLOSE @.050, ABC (C)    34       ¦ INTAKE:                     ¦

MAX LOBE LIFT, IN       .281     ¦   DURATION @ .050     204.0 ¦

LASH @ VALVE, IN        0        ¦   LOBE CENTERLINE     108.0 ¦

ROCKER ARM RATIO        1.6      ¦   GROSS VALVE LIFT    0.450 ¦

EXHAUST CAM SPECS:                 ¦                             ¦

OPEN @.050, BBC (C)     39       ¦ EXHAUST:                    ¦

CLOSE @.050, ATC (C)    -5       ¦   DURATION @ .050     214.0 ¦

MAX LOBE LIFT, IN       .296     ¦   LOBE CENTERLINE     116.0 ¦

LASH @ VALVE, IN        0        ¦   GROSS VALVE LIFT    0.474 ¦

ROCKER ARM RATIO        1.6      ¦                             ¦

ADDITIONAL CAM ADVANCE    4        ¦ LOBE SEPARATION       112.0 ¦

¦                             ¦