Re: Is a 4T60E transmission w/ # 618AHW interchangable with a 619AHW

The code 618AHW means: 61 is the transaxle, the '8' is the model year
(1998), the W at the end means it's a Hydramatic 4T60-E. The "AH" is the
actual model of the transaxle, if it's the same on both, 618AHW and 619AHW,
then one would think they are the same transaxle, just different years.

Just for comparison, my transaxle is a 615PBW (61, 1995, PB, 4T60-E), RPO
code M13, with an RPO FW9 - 3.43 final drive.

To be 100% sure, you could check out the RPO codes on both vehicles. Make
sure they are both an M13, which they should be by the info on provided in
the plates, and also check to make sure the RPO for the final drive ratio is
the same, it will be one of the following:
FPx, FQx, FRx, FVx, FWx, FYx, F1x (where 'x' is a number between 0 and 9)

Make sure they're the same on both cars.

All that being said, it is my opinion, based on the two numbers, 618AHW and
619AHW, I would say they are the same tranny and can be used without worry.