Assembly Check List, Pontiac V-8

This procedure was written to aid the assembly of a motor, and to serve as a future reference to what was, or was not, done and what parts were used.  It is primarily directed towards the Pontiac V-8, but may be of use towards other makes as well.  The steps are arranged in an order that I find logical, but by no means should the order presented be taken to be the only proper order.

No torque values or any other specifications have been provided.  Some builders may have personal specifics.  Others will need to obtain specifications from some other reference material, I strongly suggest that ALL specs be verified by a second, unrelated, reference.

All blanks may not be applicable to your specific engine.  I suggest that no blanks be left, but to enter "N/A" for "not applicable", "N/M" for "not measured", and "?" for anything you are unsure of.  If more than one person is doing the assembly, it would be beneficial if each person initialed the steps they complete in lieu of simply "checking off".

Steps/measurements that are needed for calculating compression ratio have been positively identified by a “ *** ”.  For ease of calculation, I recommend using the “Compression Calculator” that can be found on the “Restoration” page of the “Classical Pontiac” website,  www.classicalpontiac.com  .

I, the author, accept no liability for the use of this document.  By using this document, the user accepts all liability.

# Machine shop used:		Name:  ____________________________

Address:  __________________________

Phone Number:  _____________________

Head work by:			Name:  ____________________________

Address:  __________________________

Phone Number:  _____________________

Engine assembled by:		Name:  ____________________________

Address:  __________________________

Phone Number:  _____________________

Reference documents:	__________________________________

__________________________________

__________________________________

Block Prep

Block line-honed				[ Y / N ]

Amount block decked:  drvr ______;  pssgr ______

Edges filed off of sharp edges    		[ Y / N ]

All bolt-holes thread  chased      		[ Y / N ]

Oil passages rifle-brushed          		[ Y / N ]

Block washed ___ times with __________ (cleaner)

Oil galley plugs installed            		[ Y / N ]

NOTE!  double check to make sure passenger side, 			rear-most lifter supply galley plug was installed.  This 			is NOT the plug at the back of the block, rather 				accessed BY the plug at the back of the block.

Cam bearings installed 			[ Y / N ]

Brand  ________  Part #  __________

Freeze Plugs:  Brass ___  Steel ___ Sealant (if any) _______

Install dip-stick tube (middle piece, in block)     [ Y / N ]

Deck Plate used for bore/hone 		[ Y / N ]

Date section completed:  ___________________

Notes:  __________________________________________

________________________________________________

________________________________________________

Crank Prep

Oil holes chamfered				[ Y / N ]

Cross-drilled					[ Y / N ]

Journals turned:  main  _______; rod  _______

Bearing part #'s:  main  ________; rod  _______

Journals and oil holes cleaned		[ Y / N ]

Turned  _______” on main, and  _______” on rod journals

***Offset ground [ Y / N ] by _______”

Crank Prep cont.

Rear seal type/part #, sealant(s) used, installation notes:

_____________________________________________

_____________________________________________

_____________________________________________

Bearing prep (if any)   ____________________________

Assembly lube used on crank/bearings  _______________

Rear seal lubricated	[ Y / N ]  with _____________

Lube used on main cap bolts (or studs)  _______________

Main [bolts/studs], brand/part # __________/__________

Main caps [ 2 / 4 ]-bolt brand/part# ________/_________

Main bearing clearances:

1_____; 2_____; 3_____; 4_____; 5_____

Main cap torque steps  ___, ___, ___, ___, ___, ___.

NOTE:  rotate crank at end of each step to verify

there is no binding.

Main cap final torques:  1-4  ______;  rear  _____

Source(s) for torque specs  ______________________

Final torque double check:

Front	  2    thrust    4      Rear

pass. side     	___   ___   ___   ___    ___

drvr. side   	___   ___   ___   ___    ___

Crankshaft endplay measurement:  ________ "

Test fit windage tray, and turn crank one full turn to assure

there is no interference		[ Y / N ]

Date section completed:  ___________________

Notes:  __________________________________________

________________________________________________

________________________________________________

________________________________________________

Pistons and Rods

Pistons:  brand/part #  _____________________________

Piston pins:  brand/part #/weight  ____________________

***Piston valve relief/dish/dome volume _______cc’s

Piston prep (pins fitted, polishing, coatings, etc.):  ________

______________________________________________

Diameters:  piston/bore (enter "N/C" if not checked)

1 - ____/____   3 - ____/____   5 - ____/____   7 - ____/____

2 - ____/____   4 - ____/____   6 - ____/____   8 - ____/____

***Average amount bored over _________”

Rods:  brand/type  _______________________________

Shot peened  [ Y / N ] ;    Polished   [ Y / N ]

Rod bolts:  brand/part #  _____________/____________

Bolts [ torqued / stretched ] to ________

for rod resizing.  If torqued, bolts lubricated with

_____________________.

(check here if rods NOT resized ____)

Rings:  brand/part #  ____________________________

Ring gap (check here ___  if rings installed w/o measure):

Top  1 ____     3  ____    5  ____    7  ____

2  ____    4  ____     6  ____   8  ____

2nd  1 ____     3  ____    5  ____    7  ____

2  ____    4  ____    6  ____    8  ____

Ring clock position:

Driver's side - top ___o'clock;  2nd ___o'clock

Passenger    - top ___ o'clock;  2nd ___o'clock

Type of lube used on pistons skirts/rings:  _____________

Pistons and Rods cont.

Rod bearing prep:  __________________________

Lube used on rod bearings:  ___________________

NOTE:  remember protective caps for rod bolts

during piston/rod installation.

Rod bolts tightened to:

a final stretch of 0.0         ";  or

a final torque of _____, using _______ as lube.

Rod bearing final clearances:

1_____	3_____	5_____	7_____

2_____	4_____	6_____	8_____

Rotate crankshaft after each journal pair of rods/pistons are

completed to verify nothing is binding, and measure 				side clearance.

1/2  _____,  3/4  _____,  5/6  _____,  7/8  _____

***Final deck heights:

1_____	3_____	5_____	7_____

2_____	4_____	6_____	8_____

Test fit windage tray, turn crank one full revolution to

verify there is no interference -  [ Y / N ]

Oil pump:  brand/part #  _____________/___________

Oil pump driveshaft:  [ new / re-used old ]

tabs left on  ___ , or ground off ___

NOTE:  remember to install driveshaft with pump

if tabs are not ground off.

Pistons and Rods cont.

Any oil pump prep (blueprinting, etc.) ______________

Pick up screen:  pressed on ___, [welded / brazed] on ___

clearance to oil pan ______________.

Oil pump bolts torqued to _____________.

Install windage tray, and lower dipstick tube.  Test fit 	dipstick to assure proper installation.  Locking compound 	 	used on windage tray bolts - [ Y / N ]

Piston to valve clearances [ ____ not checked]

1 - ____/____   3 - ____/____   5 - ____/____   7 - ____/____

2 - ____/____   4 - ____/____   6 - ____/____   8 - ____/____

Date section completed:  ___________________

Notes:  __________________________________________

________________________________________________

________________________________________________

________________________________________________

Camshaft

Brand ___________  Grind #  ________________

Specs:  Duration (gross)____/____ (0.050")____/____

Lift _____/_____  Lobe separation angle  ______

Advance/Retard (circle one) ground into cam ___

Advance/Retard (circle one) as installed ___

Lifters:  brand  ________  part #  __________

Timing chain:  brand  _______ part #  ___________

Type of lube applied to cam lobes  ______________

Camshaft retaining plate bolts torqued to _________

Locking compound used?  		[ Y / N ]

Camshaft cont.

Install timing gears/chain, and degree cam now

Degreeing info:

____° at 0.050" before max intake lift

+ ____° at 0.050" after max intake lift

= ____ ¸ 2 = ____ = intake centerline

Fuel pump eccentric:  new ______ or used  ______

Cam bolt torqued to _______.   Type of locking compound

used ___________

Date section completed:  ___________________

Notes:  __________________________________________

________________________________________________

________________________________________________

Heads & Valvetrain

Casting # _______  Date codes drvr/pssgr ______/______

Ported by ___________________ to flow

____cfm at  ____" valve lift at ___" of water (intake)

____cfm at  ____" valve lift at ___" of water (exhaust)

Surfaced/milled __________"  Intake face milled ________"

***Chamber volumes:

1  ____      3  ____      5  ____      7  ____

2  ____      4  ____      6  ____      8  ____

Pushrod holes enlarged for 1.65 rocker arms:  [ Y / N ]

Exhaust crossover filled [ Y / N ] with ___________.

Heads & Valvetrain cont.

Valve job angles (int)  valve __________  seat _________

(exh) valve__________  seat _________

Valve guides  [iron / bronze] [new / old] [honed / knurled]

Valve seals:  intake ________;  exhaust  _________

Valve springs, brand/part #:  int __________/exh_________

installed height (int/exh) ________/________

Pressures: seat ____/____   open ____/____

Max safe valve lift ____/____

Coil bind at ____/____ ;  Retainer to seal  ____/____

Retainers:  brand/part # _______/______; material _______

Locks:  brand/part #       ________/______ [ 7 / 10 ]

Rocker arms:  brand ________; part # _______; ratio ____

Rocker studs:  brand ________; part # _______; size ____

torqued to _________ with locking compound [ Y / N ]

Adjusting nuts:  brand ________; part # _______

Pushrods:  brand ________; part # _______; length _____

Head fasteners [bolts/studs]:  brand _______; part # ______

if studs used, installed with locking compound [ Y / N ]

Head gasket:  brand ________; part # __________

check for imperfections - drvr[ Y / N ]; pssgr[ Y / N ]

*** Head gasket compressed thickness 0.0____”

Block deck cleaned with solvent [ Y / N ]

Cylinder deck cleaned with solvent [ Y / N ]

Lube used on head fastener threads ________________

Head bolts torqued in steps of ___, ___, ___, ___, ___,

___ to a final torque of ____.  Reference for torque

sequence is _____________________________

Date section completed:  ___________________

Notes:  __________________________________________

________________________________________________

________________________________________________

Buttoning it up

Install new seal on timing cover.  Seal part #  ___________

Seal lubricated with ___________

____Timing cover installed.  Anti-seize on bolts [ Y / N ]

____Inspect harmonic damper for damage (i.e. cracks

around keyway, rubber deterioration, etc.)

Damper (brand/part# ________/________)torqued to _____

SFI approved [ Y / N ]

____Test fit oil pan

____Install oil pan & gaskets.  Sealant used __________

(Flywheel / flexplate) brand/part # _________/________

SFI approved [ Y / N ]

bolts torqued to ________

Locking compound used [ Y / N ]

NOTE:  installation with motor on engine stand

may not be possible with all stands.

____ Install lifters,  pre-lubed  [ Y / N ] with ____________

____ Install pushrods & rocker arm assemblies

Rocker arm ( lash / preload ) adjusted to ______________

____ Install valley pan.  New PCV grommet used [ Y / N ]

Date section completed:  ___________________

Notes:  __________________________________________

________________________________________________

________________________________________________

________________________________________________

At this point, I consider the engine "built".  Intake, distributor, exhaust manifold.... and the remaining installations are rather straight forward, and do not fall in the scope of this procedure.