Speedometer Information

The following formula will work for any GM, Ford or Chrysler vehicle and is very useful when there is no established baseline from which to work from such as replacing a transmission in a vehicle that was purchased without a drive line. The accompanying charts show what gears were manufactured for each type transmission. No part numbers are given because they are often changed by the OEM. See your local dealer for the appropriate part number after determining which gears you require.

Note: Drive gear refers to the gear that is on the transmission output shaft. The driven gear is located in a removable housing usually in the tailhousing of the transmission. Be aware that some transmissions will have different driven gear housings depending on the tooth count of the driven gear.

| # Drive Teeth X Axle Ratio X Tire Rev. per Mile | = # Driven Teeth |
| --- | --- |
| 1001 |  |

Use the following formula to calculate the Tire Rev. per Mile for the above formula.

| Tire Rev. per Mile = | 20168 |
| --- | --- |
|  | Tire Diameter (inches) |

Try this handy custom calculator. It will do the work for you.

Top of Form 1

| # of Drive Teeth: Axle Ratio: Tire Diameter: | # of Driven Teeth: |
| --- | --- |

Bottom of Form 1

200-4R | TH-350 | TH-400 | 700-R4 | Torqueflite

GM SPEEDOMETER GEAR INFORMATION

200-4R Speedometer Gears

| Drive Gears | Driven Gears |
| --- | --- |
| 10 teeth - green | 18 teeth - brown |
| 11 teeth - orange* | 19 teeth - white |
| 12 teeth - red | 20 teeth - green |
| 13 teeth - white | 22 teeth - gray |
|  | 26 teeth - brown |
|  | 27 teeth - black |
|  | 28 teeth - yellow |
|  | 29 teeth - green |
|  | 30 teeth - blue |
|  | 31 teeth - white |
|  | 35 teeth - orange |
|  | 40 teeth - black |
|  | 42 teeth - green |
|  | 43 teeth - purple |
|  | 44 teeth - dark gray |
|  | 45 teeth - light blue |

Notes:

- All 200-4R transmissions use a 1" diameter steel driven gear housing.

- The drive gear is part of the governor assembly which is located inside the oil pan. The gear can be replaced separately by removing the retainer clip on the governor shaft.

* - This gear is shipped in all TCI Street Fighter 200-4R transmissions (Original to Turbo Buick applications).

TH-350 Speedometer Gears

| Drive Gears | Driven Gears |
| --- | --- |
| 7 teeth - pink | 18 teeth - brown |
| 8 teeth - orange | 19 teeth - natural |
| 9 teeth - green | 20 teeth - blue |
| 10 teeth - red or purple | 21 teeth - red |
| 17 teeth - red | 22 teeth - gray |
| 18 teeth - blue | 34 teeth - light green |
| 19 teeth - yellow | 35 teeth - orange/pink |
| 20 teeth - brown | 36 teeth - white |
|  | 37 teeth - red |
|  | 38 teeth - blue |
|  | 39 teeth - brown |
|  | 40 teeth - black |
|  | 41 teeth - yellow |
|  | 42 teeth - green |
|  | 43 teeth - purple |
|  | 44 teeth - dark gray |
|  | 45 teeth - light blue |

Notes:

- Chevrolet transmissions use a 1" diameter steel driven gear housing located in the tailhousing on all two-wheel drive applications.

- Buick, Oldsmobile & Pontiac transmissions use a 2.078" diameter driven gear housing. Two housings, each made from either aluminum or plastic, are available for driven gears with 34 or more teeth. One fits gears from 34 - 39 teeth and the second one fits gears from 40 - 45 teeth.

TH-400 Speedometer Gears

| Drive Gears | Driven Gears |
| --- | --- |
| 7 teeth - red | 16 teeth - (truck) |
| 8 teeth - black | 17 teeth - (truck) |
| 15 teeth - brown or gray* | 18 teeth - (truck) code H |
| 16 teeth - orange | 19 teeth - (truck) code J |
| 17 teeth - red | 20 teeth - (truck) code K |
| 18 teeth - yellow or blue | 21 teeth - (truck) code L |
| 19 teeth - yellow or white | 22 teeth - (truck) code M |
| 20 teeth - green | 34 teeth - light green |
| 21 teeth - black | 35 teeth - pink |
|  | 36 teeth - white |
|  | 37 teeth - red |
|  | 38 teeth - blue |
|  | 39 teeth - brown |
|  | 40 teeth - black |
|  | 41 teeth - yellow |
|  | 42 teeth - green |
|  | 43 teeth - purple |
|  | 44 teeth - dark gray |
|  | 45 teeth - light blue |

Notes:

- Two aluminum housings are available for driven gears with 34 or more teeth. One fits gears from 34 - 39 teeth and the second one fits gears from 40 - 45 teeth.

- The driven gear housings go in the case just forward of the tailhousing.

* - This gear is shipped in all TCI Street Fighter TH-400 transmissions.